AES MINI PROJECT

Name:Ravina yadav

Roll No. : 32

Subject:Msc-C.S part-1

Aim: Using Arduino display temperature on LCD

Soln:

Step1: Open tinkercad and click on circuits and add new circuit.

Step2: Now click on search box and type name of the components needed, and drag the component on the screen.

Step 3: Simillarly drag all the needed components on the screen like breadboard,LCD,potentiometer,resistor and temperature sensor like tmp36(DHT is not available tinkercad so we will use tmp36.)

Step4:Now we need to make all the connections, ardunio LCD connection and tmp36 with ardunio.

- Connect 5v of ardunio with 5v of the LCD

- Now we will connect negative pin (GND) with Gnd

- Connect potentiometer positive,negative terminal with gnd and 5v of the LCD and connect middle of the potentiometer with the Vo pin of the LCD.

- Connect LCD anode with the 5v pin of the Arduino and cathode pin with resitor and the resistor with the GND of the Arduino.

- Rs pin of the LCD with the 8 of Arduino

- Rw pin with GND

- E pin with 7

- D4 pin the 6

- D5 pin 5

- D6 pin 4

- D7 pin 3

- Now we need to connect temperature sensor, power pin of tmp36 will be connected to vcc of ardunio and negative pin will be connected to the GND of arduino.

- And output pin will be connected to A0 of arduino.

Step4.After making all the connections we need to write the code and start simulation in tinkercad.

Code:

#include "LiquidCrystal.h"

LiquidCrystal lcd(8,7,6,5,4,3);

int sensorPin = 0;

void setup()

{

Serial.begin(9600);

lcd.begin(16,2);

}

void loop()

{

int reading = analogRead(sensorPin);

// measure the 5v with a meter for an accurate value

//In particular if your Arduino is USB powered

float voltage = reading * 4.68;

voltage /= 1024.0;

// now print out the temperature

float temperatureC = (voltage - 0.5) * 100;

Serial.print(temperatureC);

Serial.println(" degrees C");

lcd.setCursor(0,0);

lcd.print("Temperature Value ");

lcd.setCursor(0,1);

lcd.print(" degrees C");

lcd.setCursor(11,1);

lcd.print(temperatureC);

delay(100);

}

Output: